SXOPSUM.DOC        PIC16C5x INSTRUCTION SET SUMMARY WITH SCENIX EXTENSIONS        AUG-05-99

Mnemonic,      Description                         Cycles    12-Bit Opcode      Status

Operands                                           /Turbo   MSB         LSB     Affected  Notes

===============================================================================================

ADDWF   f,d    Add W and f                           1      0001  11df  ffff    C,DC,Z    1,2,4

ANDWF   f,d    AND W with f                          1      0001  01df  ffff    Z         2,4

CLRF    f      Clear f                               1      0000  011f  ffff    Z         4

CLRW    -      Clear W                               1      0000  0100  0000    Z

COMF    f,d    Complement f                          1      0010  01df  ffff    Z         2,4

DECF    f,d    Decrement f                           1      0000  11df  ffff    Z         2,4

DECFSZ  f,d    Decrement f, Skip if 0                1(2)   0010  11df  ffff    None      2,4

INCF    f,d    Increment f                           1      0010  10df  ffff    Z         2,4

INCFSZ  f,d    Increment f, Skip if 0                1(2)   0011  11df  ffff    None      2,4

IORWF   f,d    Inclusive OR W with f                 1      0001  00df  ffff    Z         2,4

MOVF    f,d    Move f                                1      0010  00df  ffff    Z         2,4

MOVWF   f      Move W to f                           1      0000  001f  ffff    None      1,4

NOP     -      No Operation                          1      0000  0000  0000    None

RLF     f,d    Rotate left f through Carry           1      0011  01df  ffff    C         2,4

RRF     f,d    Rotate right f through Carry          1      0011  00df  ffff    C         2,4

SUBWF   f,d    Subtract W from f                     1      0000  10df  ffff    C,DC,Z    1,2,4

SWAPF   f,d    Swap nibbles of f                     1      0011  10df  ffff    None      2,4

XORWF   f,d    Exclusive OR W with f                 1      0001  10df  ffff    Z         2,4

-----------------------------------------------------------------------------------------------

BIT-ORIENTED FILE REGISTER OPERATIONS

-----------------------------------------------------------------------------------------------

BCF     f,b    Bit Clear f                           1      0100  bbbf  ffff    None      2,4

BSF     f,b    Bit Set f                             1      0101  bbbf  ffff    None      2,4

BTFSC   f,b    Bit Test f, Skip if Clear             1(2)   0110  bbbf  ffff    None

BTFSS   f,b    Bit Test f, Skip if Set               1(2)   0111  bbbf  ffff    None

-----------------------------------------------------------------------------------------------

LITERAL AND CONTROL OPERATIONS

-----------------------------------------------------------------------------------------------

ANDLW   k      AND Literal with W                    1      1110  kkkk  kkkk    Z

CALL    k      Call subroutine                       2/3    1001  kkkk  kkkk    None      1

CLRWDT  k      Clear Watchdog Timer, Pre=0 TO=1 PD=1 1      0000  0000  0100    TO,PD

GOTO    k      Unconditional branch                  2/3    101k  kkkk  kkkk    None

IORLW   k      Inclusive OR Literal with W           1      1101  kkkk  kkkk    Z

MOVLW   k      Move Literal to W                     1      1100  kkkk  kkkk    None

OPTION  k      Load OPTION register with W           1      0000  0000  0010    None

RETLW   k      Return (fr subr) move Literal to W    2/3    1000  kkkk  kkkk    None

SLEEP   -      Power down, WDT=0 Pre=0 TO=1 PD=0     1      0000  0000  0011    TO,PD

TRIS    f      Move W into Port Control Register     1      0000  0000  0fff    None      3

XORLW   k      Exclusive OR Literal to W             1      1111  kkkk  kkkk    Z

-----------------------------------------------------------------------------------------------

SCENIX EXTENSIONS

-----------------------------------------------------------------------------------------------

BANK    n      Write n into FSR7, FSR6 and FSR5      1      0000  0001  1nnn    None         5

IREAD   -      Read word at (MODE:W) into MODE:W     1/4    0000  0100  0001    None

MODE    n      Write n into MODE register            1      0000  0101  nnnn    None

MOVMW   -      Move MODE bits to W, High nibble=0    1      0000  0100  0010    None

MOVWM   -      Move W to MODE register               1      0000  0100  0011    None

PAGE    n      Write n into PA2, PA1 and PA0         1      0000  0001  0nnn    PA2,PA1,PA0  5

RET     -      Return without affecting W            2/3    0000  0000  1100    None

RETI    -      Ret. from int. Pop PC W STATUS FSR    2/3    0000  0000  1110    All but TO,PD

RETIW   -      RETI and add W to RTCC                2/3    0000  0000  1111    All but TO,PD

RETP    -      Return, W unaffected, Pop PA2 PA1 PA0 2/3    0000  0000  1101    PA2,PA1,PA0

Notes:

1.  The 9th bit of the program counter will be forced to a '0' by any instruction that writes

to the PC except GOTO.

2.  When an I/O register is modified as a function of itself (e.g: MOVF PORTB,1 ;Test), the

value used will be the value present on the pins themselves. For example, if the data

latch is '1' for a pin configured as an input but it is driven low by an external device

then the data used will be '0'.

3.  The instruction TRIS f (where f = 5, 6 or 7) causes the contents of the W register to be

written to the port control register selected by the MODE register. If the MODE register

= 0Fh, W will be written to the tristate latches of PORTA, B or C respectively. A '1'

forces the pin to a high impedance state and disables the output buffers.

4.  If this instruction is executed on the RTCC register (and, where applicable, d=1), the

prescaler will be cleared if it is assigned to RTCC rather than the WDT.

5.  If this instruction is immediately preceded by a skip instruction (such as BTFSS or

DECFSZ, and the condition is true) then both this instruction and the next instruction

will be skipped and the operation will consume 3 cycles.

b   =  bit 0..7 in f

C   =  carry status bit

d   =  destination: W or f (d=0 for W, d=1 for f)

DC  =  digit carry status bit

FSR =  file select register

f   =  file register (RAM)

k   =  8-bit constant (literal)

n   =  constant

PAn =  page select status bits (n = 0,1,2)

PD  =  power-down (sleep-mode) status bit (true low)

Pre =  prescaler (cleared if assigned to WDT)

TO  =  watchdog timer time-out status bit (true low)

W   =  working register (accumulator)

WDT =  watchdog timer

Z   =  zero status bit

-Loren Blaney