SXSET.DOC     AUG-05-99

This is the Scenix instruction set grouped by function. These mnemonics

follow the Microchip standard rather than the Parallax standard used by

Scenix.

ADDWF  f,d  C,DC,Z

SUBWF  f,d  C,DC,Z

ANDWF  f,d  Z              ANDLW  k    Z

IORWF  f,d  Z              IORLW  k    Z

XORWF  f,d  Z              XORLW  k    Z

MOVWF  f    -              MOVLW  k    -

MOVF   f,d  Z

CLRF   f    Z              CLRW   -    Z

COMF   f,d  Z

DECF   f,d  Z              INCF   f,d  Z

DECFSZ f,d  -              INCFSZ f,d  -

RLF    f,d  C              RRF    f,d  C

SWAPF  f,d  -

BCF    f,b  -              BSF    f,b  -

BTFSC  f,b  -              BTFSS  f,b  -

GOTO   n    -  (512)     * PAGE   n    PA2:PA0  (8)

CALL   k    -  (256)

* RET    -    -              RETLW  k    -

* RETP   -    PA2:PA0

* RETI   -  All but TO,PD  * RETIW  -  All but TO,PD

* MOVWM  -    -            * MODE   n    -  (16)

* MOVMW  -    -

* IREAD  -    -

* BANK   n    -  (8)

CLRWDT -    TO,PD

SLEEP  -    TO,PD

OPTION -    -

TRIS   f    -

NOP    -    -

*   =  new instruction not in PIC16C5x

b   =  bit 0..7 in f

C   =  carry status bit

d   =  destination: W or f (d=0 for W, d=1 for f)

DC  =  digit carry status bit

f   =  file register (RAM)

k   =  8-bit constant (literal) (256)

n   =  constant

PAn =  page select status bits (n = 0,1,2)

PD  =  power-down (sleep-mode) status bit (true low)

TO  =  watchdog timer time-out status bit (true low)

W   =  working register (accumulator)

Z   =  zero status bit

There is some confusion as to what the new instructions added by Scenix

actually do. Here are some details.

MODE n - The four literal bits of "n" are loaded into the MODE register.

Status affected: None

Encoding: 0000 0101 nnnn

Words: 1

Cycles: 1 (for both PIC16C5x compatible mode and turbo mode)

Example: MODE 0Eh       ;After instruction, MODE register contains 1110

MOVMW - Copies the four bits in the MODE register into W. The high four

bits of W are zeroed.

MOVWM - Copies the four low bits of W into the MODE register.

IREAD - Used to read 12-bit instruction memory. The contents of address

(MODE:W) is copied into MODE:W. In other words: First store the high four

bits of the address to be read into the MODE register and the low eight

bits of the address into the W register. After the IREAD instruction is

executed, the high four bits of the addressed location are in MODE and

the low eight bits are in W.

The IREAD instruction has the interesting property that it takes the same

amount of time to execute regardless of the TURBO_ bit in FUSE (DEVICE

TURBO). It takes 4 cycles in turbo mode and 1 cycle in compatible mode.

RETI - Return from interrupt. The PC, W, STATUS and FSR registers are

restored to the values they had when the interrupt occurred. Shadow

registers are used to hold these values; the subroutine return address

stack is not used. Thus subroutines can be called to a depth of eight

levels while interrupts are enabled.

RETIW - Same as RETI but adds the contents of W to RTCC without affecting

the prescaler. This provides a periodic interrupt that can be adjusted by

256 steps (minus the time it takes to execute the interrupt routine).

RET - Same as RETLW but the W register is not affected. (BEWARE: MPASM

without warning assembles RETURN as RETLW 0.)

RETP - Same as RET but the return address bits 11, 10 & 9 (on the stack)

are written to the page-select bits PA2, PA1 & PA0 in the STATUS

register. Thus the page-select bits are properly set to the page being

returned to.

PAGE n - The three literal bits of "n" are loaded into the page-select

bits PA2:PA0 in the STATUS register. This normally prepares a GOTO or

CALL across a page boundary. PA2 is only used in chips with 4K of

instruction memory (SX48BD and SX52BD).

BANK n - The three literal bits of "n" are loaded into the bank-select

bits (the high three bits) in the FSR register. In PIC16C54 compatible

mode, only one bank is enabled and the three high bits in FSR are always

set. (INCFSZ FSR still skips when FSR is initially 0FFh even though the

result is 0E0h.)

PAGE and BANK cause an unusual operation when preceded by a skip instruc-

tion such as BTFSS and INCFSZ. Both the PAGE or BANK instruction and the

following instruction are skipped, and the operation takes 3 cycles. If

the following instruction happens to be another PAGE or BANK then it is

also skipped along with the next instruction, and the operation takes 4

cycles. Any number of PAGE and BANK instructions can be skipped in this

manner. Interrupts don't interfere with this operation.

SUBWF - If the CF_ bit in FUSEX is 0 (DEVICE CARRYX), the complement of C

is subtracted (borrowed) from the result. The intent of the CF_ bit is to

make multiprecision adds and subtracts easier, as shown here:

;FF:= FF + GG with correct carry out (CF_ is 0)

MOVF    G,W             ;add low bytes

BCF     STATUS,CF       ;clear carry

ADDWF   F

MOVF    G+1,W           ;add high bytes and carry

ADDWF   F+1

;FF:= FF - GG with correct carry out (CF_ is 0)

MOVF    G,W             ;subtract low bytes

BSF     STATUS,CF       ;set carry

SUBWF   F               ; F - G

MOVF    G+1,W           ;subtract high bytes

SUBWF   F+1

The following routines accomplish the same thing (although with an extra

instruction), and they're compatible with the PIC16C5x:

;FF:= FF + GG with correct carry out (CF_ is 1)

MOVF    G,W             ;add low bytes

ADDWF   F

MOVF    G+1,W           ;get ready to add high bytes

BTFSC   STATUS,CF       ;skip if there was no carry into high byte

INCFSZ  G+1,W           ;else add in carry; if result is 0 then high

ADDWF   F+1             ; byte doesn't change and CF is still set

;FF:= FF - GG with correct carry out (CF_ is 1)

MOVF    G,W             ;subtract low bytes

SUBWF   F

MOVF    G+1,W           ;get ready to subtract high bytes

BTFSS   STATUS,CF       ;skip if there was no borrow from high byte

INCFSZ  G+1,W           ;else increase amount to subtract by 1; if it's

SUBWF   F+1             ; 0 then high byte doesn't change, nor does CF

Since the carry-out is correct in all four of these routines, they can

easily be extended for more bytes of precision.

The PIC16C5x can do a double precision add in five instructions if there

is a location set aside to hold a zero. Unfortunately, this can't be

extended to more than two bytes of precision.

;FF:= FF + GG with correct carry out (CF_ is 1)

MOVF    G,W             ;add low bytes

ADDWF   F

RLF     ZERO,W          ;shift in carry forming either 01h or 00h

ADDWF   G+1,W           ;add carry and high bytes

ADDWF   F+1

CLRWDT and SLEEP only clear the prescaler if it is assigned to the

watchdog timer (OPTION bit 3 set). Conversely, writing to RTCC only

clears the prescaler if it is assigned to the RTCC (OPTION bit 3 clear).

MIWU

WKPND should be cleared before enabling WKEN interrupts, otherwise a

spurious interrupt will occur.

The interrupt service routine must clear WKPND to enable additional

interrupts on the MIWU pin.

-Loren Blaney