Linistepper with aligned transistor’s

Rev. 1.0

This Eagle layout was made over the posix version of Linistepper.

Tanks to him for the outstanding work he has done.

Before of everything and because - as I write this - the lovely Microsoft Word is trying to fix the mistakes I do, I should warn you that my original language is Spanish so the spelling will be possibly a little bit wrong.

And what the hell is this?

This is a version of the well known Linistepper.

Posix did in is moment the schematic and board on Eagle Layout Editor for all of those who wish for it.

Just because I notice that some people didn’t like the idea of placement of TIP122 NPN transistors, and also myself found in the problem of choosing some good, adaptable and     -Why not- good looking heat sink, after I decide to built this driver.

Let’s not forget the obvious fact that you can always buy the original board (with aligned transistors) and the parts to solder for almost nothing of money. But some people like me are far away from any source of it. Also I have discovered the nice hobby of building my own pcb’s using the tonner transfer method witch some of you know.

That’s how this started.

Note: For those things of destiny, the heat sink’s I choose for my drivers was a large, big and nice standard type of dissipator used on Pentium 4 computers and therefore easy to find. Actually you can buy those on a lot of places (Meaning electronic part supplier stores, computer stores, and even better, used computer part suppliers were you will find the original heat sink which comes with the Intel motherboard’s for the price of a cigarette box, if you have some look (Like my).

Changes made on the board

If you look at the board - Assuming that you: Know what Eagle is for, have it, use it, know it…- you will notice that the parts are almost the same except for the change of two groups of it:

- Part’s X2, X3, X4 described as “W237-102” from the library “con-wago-500”, are now simple pinheader’s; for ground, step and direction signal from pc are better (At list like I see it) than what wore before for a bunch of reasons and specially for me, after I decide to connect them directly to the pc parallel port (Some people don’t recommend this, so be aware).

- The need for space on the tip122 transistor zone, make my change the motor coils connection type proposed by Posix for another (2 lines Pinheaders). Let my say that this kind of electronic part can be found on any small electronic part store for little money and lots of pins to work with, enough for a numerous electronic projects. Also the stepper motors I have came with this kind of connection.

After that, the job was mostly about replacement of parts location, re-routing and more robust traces to ensure that the tonner transfer will by well made and also avoiding surprises on chemical attack.

On the ripping and re-routing process I was forced to add some top wires to chomp signals on the board (I don’t now how to say it in English; I think they are…jumpers?) so I will appreciate if some expert take a look at some possible mistake I made on this part.

Finally about the changes made section, there are no more surprises to look at, and I will love to listen any suggestions or improvements and things to do again (I’m secure there are a few, but nothing that live the board impractical).

What’s in the box

(What is in this zip file)

- Readme.doc – This same file 

- Bottom.bmp – Picture of the cupper side of the project. There are a few things improved here from the “image export” function of Eagle Layout Editor, using Photoshop. They are slight and only because of my perfectionist soul and not worth to mention.

- Top.bmp – Picture of the outer side of the project. Again there are a few things improved here from the “image export” function of Eagle Layout Editor and now the changes made worth to be mention and remark: Usually, if you add the “tvalue” and “bvalue” on the display layers window configuration – left side of Eagle – to print your project like I do, it is a known overwriting of numbers and letters between symbols, so I have made the job of taking this and improve it not only in the program, but also afterword’s using Paint and Photoshop to move, change, erase and add the needed ones. Notice specially that I erase the letters in blue from the authors, not because they didn’t deserve the place, but for reasons of dirty look on the final artwork; I want to emphasize this, as there was not intention to discard their merit. The reason for erasing it, is since I didn’t now how to move it to a proper place in the board (Talking about the blue letters).

- Top mirrored.bmp – Needed for printing and tonner transfer. This is the one included in the .doc file described below. I strongly suggest you to use this “top mirrored.bmp” for your project since is a well made job.

- Linistepper with aligned transistor’s ready for glossy paper printing.doc – This is the file ready to print in glossy paper, and you will see that they are 4 possible board’s to make from one print; Well, if you need only 1 board for example, is not a bad idea to print the all thing just in case that the first attempt of tonner transfer didn’t work well. (Anyways, that’s a manner you can decide without my to suggest you what to do…like if you be my son, right?).

- Linistepper posix with aligned transistors.brd – Board File for Eagle Layout Editor

- Linistepper posix with aligned transistors.sch – Schematic file for Eagle Layout Editor

------------------------------------------------------------------------------------------------------------

So as final words to say about this, I can say that this is my first collaboration to any forum on this world, after ears of socking the knowledge of many people which have save my ass a lot of times so I hope this equal the balance a little beat and promise from now on to help more instead of download, and download, and download…

Patricio Oholeguy

Argentina, Buenos Aires

08/17/2009

Rev. 1.0

------------------------------------------------------------------------------------------------------------