SIM80

An 8080 simulator

for the 6809

Revised: 30-Jul-90

Copyright 1983,1990 Dave Dunfield

All rights Reserved

SIM80                                                            Page: 1

1. INTRODUCTION

SIM80 is an simulator for the Intel 8080 processor,  which runs on

the motorola 6809 under CUBIX. It allows software written for an 8080

to be run on the 6809.

1.1 DIFFERENCES

SIM80 exactly emulates the 8080 instructions and  architecture,

except for the following:

1) SIM80 executes slower than a hardware 8080.

2) The parity flag is not supported, along with the parity testing

instructions (JPE, JPO, CPE, CPO, RPE, RPO). Most 8080 software

does not use the parity flag,  and it was decided emulating the

parity flag was not worth  the  reduction  in  emulation  speed

which would be incurred,  as this is an operation not  directly

supported on the 6809.

3)  The I/O instructions 'IN' and 'OUT'  are not supported because

the 6809 has no I/O address space.  The  'OUT'  instruction  is

used to interface to the 6809 operating system (Below).

4)  The enable/disable interrupts instructions 'EI'  and 'DI'  are

not supported due to lack of  physical  interrupt  lines  on  a

simulated processor.

5) The 8080 restart instructions are not supported.

If SIM80 encounters a halt instruction, and invalid opcode,  or

an unsupported  opcode,  it  issues  an  appropriate  message  and

terminates, returning to the 6809 DOS.

SIM80                                                            Page: 2

2. USING SIM80

To run an 8080 program under SIM80,  first download the 8080  code

into a file with the type '.80'.  Then,  to execute the 8080 program,

type:  ' SIM80 <filename> ' where <filename>  is the name of the file

containing the 8080 code.

The 'DE'  register pair of the emulated 8080 processor will be set

to point to the command line after the filename, allowing operands to

be parsed by the program under simulation.

The  'SP'  register of the emulated 8080 processor will be set  to

point to the top of available memory.

3. INTERFACE TO 6809 OPERATING SYSTEM

SIM80 supports an interface to the 6809 DOS (CUBIX) via the unused

8080 'OUT' instruction. When an OUT instruction is encountered by the

simulator,  the 8080 registers are mapped to the  6809  registers  as

follows:

8080         6809

B-C   <==>    A-B (D)

D-E   <==>    Y

H-L   <==>    X

SIM80 then executes the system call indicated by the  port  number

specified by the OUT instruction. After the system call is performed,

the processor registers are mapped back,  the return  code  from  the

system call is placed  in  the  8080  'A'  register,  and  simulation

resumes.

SIM80                                                            Page: 3

3.1 Debug mode

The simulator has a  built  in  8080  debugger,  which  can  be

invoked by the '/DEBUG' qualifier on the SIM80 command.

Eg:  ' SIM80/DEBUG <filename> '

If this qualifier is used,  SIM80 will load the  file,  display

the starting address, and then prompt with '8080> '. The following

commands may now be entered:

SET    TRACE                     - Enable/Disable TRACE mode.

NOTRACE

PSW <value>

BC  <value>

DE  <value>               - Set contents of 8080 Registers.

HL  <value>

SP  <value>

PC  <value>

BRKPT <0-7> <address>     - Set breakpoint.

DISPLAY                          - Displays registers and breakpoints.

MEMORY <start addr> <end addr>   - Display memory.

STORE <address> <byt1> <byt2>... - Store into memory.

EXIT                             - Exit debugger.

GO                               - Begin simulation, If TRACE is

enabled, then address, opcode

and registers  are  displayed

as simulation  occurs.  <ESC>

key can be used to abort.

STEP                             - Similar to  go,  but  prompts

with ':' and waits for  space

before simulating  each  8080

instruction. <CR> exits.

NOTE: SIM80 executes considerably SLOWER when using the '/DEBUG'

qualifier.

SIM80

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

1.1 DIFFERENCES                                                     1

2. USING SIM80                                                         2

3. INTERFACE TO 6809 OPERATING SYSTEM                                  2

3.1 Debug mode                                                      3