CUBIX

System Porting

Guide

Revised: 30-Jul-90

Copyright 1983,1990 Dave Dunfield

All rights Reserved

CUBIX System Porting Guide                                       Page: 1

1. INTRODUCTION

The CUBIX operating system is designed to be ported at the  object

(minor port), or source code (major port) level.

A minor port may be performed whenever the target system  conforms

to the standard CUBIX memory map. The development system used must be

capable of linking a ROMable image. This memory map is as follows:

$0000-$1FFF - I/O devices (Incl. memory mapped video etc).

$2000-$DFFF - Random Access Memory.

$E000-$FFFF - CUBIX Operating System ROM

A minor port is accomplished  by  writing  the  I/O  drivers,  and

linking those drivers into the CUBIX image.  The  system  application

and utility programs are provided on a CUBIX format 5.25"  disk,  and

are ready to use.

A minor port may also be performed from a CUBIX distribution  ROM,

as described in the section on ROM porting.

A major port is required  whenever  the  target  system  does  not

conform to the above memory map,  or the develoment system used  does

not have the capability to link absolute location ROMable images.

A major port is accomplished  by  writing  the  I/O  drivers,  and

including them in the CUBIX operating system source  code,  which  is

then assembled into a ROMable image.  The memory  addresses  used  by

CUBIX may be altered at this time.

The  system  application  and  utility  programs  must   also   be

assembled, to properly locate them in the target system memory map.

In all cases,  CUBIX is placed in an 8K (2764 or similar)  ROM and

placed at the top of the 6809 memory map  (Locations $E000 to $FFFF).

The CHECKSUM of the rom must be stored as a 16 bit value in the first

two rom locations.

The ROM CHECKSUM is calculated as the 16 bit  sum  of  all  8  bit

values from offset $0002 to the end of the rom  (Offset $1FFF).  This

16 bit checksum must be stored in the rom with the  most  signifigant

byte at offset $0000, and the least signifigant byte at offset $0001.

CUBIX System Porting Guide                                       Page: 2

2. HARDWARE INITIALIZATION

The I/O  drivers  must  include  a  routine  called  HWINIT  which

establishes the CUBIX system default parameters, initializes hardware

devices,  installs all device drivers,  and prepares the console  for

the CUBIX startup messages.

2.1 Initializing CUBIX

When HWINIT is called, the 6809 'Y' register is set to point to

an internal table which must be filled in by HWINIT. The format of

this table is as follows:

OSRAM   EQU        *        Address of OS initialized ram (passed in 'Y').

RMB        7        Default DCB for drive A

RMB        7        Default DCB for drive B

RMB        7        Default DCB for drive C

RMB        7        Default DCB for drive D

RMB        1        Default console input device (0-7)

RMB        1        Default console output device (0-7)

RMB        16       Serial device input driver vectors (8 words)

RMB        16       Serial device output driver vectors (8 words)

RMB        2        Disk HOME HEAD routine.

RMB        2        Disk READ SECTOR routine.

RMB        2        Disk WRITE SECTOR routine.

RMB        2        Disk FORMAT routine.

RMB        2        SWI  handler vector address

RMB        2        SWI2 handler vector address

RMB        2        SWI3 handler vector address

RMB        2        IRQ  handler vector address

RMB        2        FIRQ handler vector address

RMB        2        NMI  handler vector address

RMB        1        Error messages enabled flag

RMB        1        SSR Debug enabled flag

RMB        1        Command file trace enabled flag

RMB        1        Default drive (0-3)

RMB        8        Default directory prefix

RMB        1        System drive (0-3)

RMB        8        System directory prefix

Notes  1) CYL, HEAD, and SEC values of DCB's should be set to zero.

2) Default console input device is usually 1

3) Unused serial device drivers should be set to zero.

4) SWI vector should be set to execute the CUBIX SSR handler,

which is located at address $E00B.

5) Unused SWI2, SWI3, IRQ, FIRQ, NMI vectors should point

to an 'RTI' instruction.

6) Usual settings of ERROR, TRACE and DEBUG are $FF, 0, 0.

($FF = ENABLED, 0 = DISABLED).

7) Drives are offset zero ('A'=0, 'B'=1, 'C'=2, 'D'=3).

8) Unused portions of directory prefix's should be padded with

zeros to the right.

CUBIX System Porting Guide                                       Page: 3

2.2 Initializing Hardware Devices

HWINIT  must  initialize  the  hardware  in  the  system,  this

includes such things as the disk drive controller, serial devices,

video displays,  and any other devices which must be available for

CUBIX to operate.

2.3 Initializing the Console

For systems using a serial  terminal  as  the  console,  HWINIT

should output a line feed,  and carraige  return  to  the  console

device to establish the cursor at the left hand margin.

For systems using an integrated memory  mapped  video  display,

HWINIT should perform a  clear  screen  operation,  and  home  the

cursor.

3. SERIAL DEVICE DRIVERS

The serial device drivers installed by HWINIT are free to use  the

6809 'B'  and 'X'  registers without saving them.  When the driver is

called,  the  'B'  register will  contain  the  CUBIX  device  number

multiplied by two.

3.1 Input Drivers

Input device drivers should  test  for  a  character  from  the

device, and return with the character in the 'A' register, and the

'Z'  condition flag set if a character was found.  If no character

is found, the 'A' register should be loaded with $FF,  and the 'Z'

flag cleared.

3.2 Output Drivers

Output drivers should write the character passed  in  the  6809

'A'  register to the device.  It should not return until this  has

been done.

3.3 NULL Device Driver

The NULL DEVICE  (device 0)  drivers are easily implemented  as

follows:

RDNULL  LDA     #$FF    INDICATE NO CHARACTER READY

WRNULL  RTS             RETURN TO CALLER

3.4 Similar Devices

When several similar devices are available,  the device drivers

usually load the 6809 'X' register with the address of the device,

and then jump to a common handler.

The  standard  'STTY'  utility  supplied  with   CUBIX,   which

functions with 6551 uarts,  examines the first instruction of  the

input driver, looking for an 'LDX' instruction,  from which it can

determine the address of the device.

CUBIX System Porting Guide                                       Page: 4

If no 'LDX' is found, STTY aborts with the error message:

'Device has fixed comunication format.'

4. DISK DEVICE DRIVERS

The disk drivers installed by HWINIT are free to use the 'A', 'B',

'X' and 'Y' registers without saving them. When a routine is invoked,

the 6809 'U' register will be set to point to the drive control block

(DCB).  See the CUBIX system programming guide for a  description  of

the DCB.

If an operation is unsuccessful,  the disk  driver  should  return

with the  'Z'  flag cleared,  and a  value  in  the  'A'  accumulator

indicating the proper error message to display as follows:

0 - 'Disk system error'

1 - 'Disk format error'

2 - 'Bad sector'

3 - 'Sector not found'

4 - 'Disk write protected'

If 'A' is set to any other value, it is treated as zero (0).

The following disk routines are required:

4.1 Home head

The  home  routine  resets  the  head  of  the  physical  drive

indicated in the DCB to track zero.

4.2 Read a sector

The read routine reads sector (512 bytes) indicated by the CYL,

HEAD and SEC entries in the DCB from the indicated physical drive,

and places it in memory at the address passed in the 'X' register.

The routine must perform a seek operation to position the head  to

the correct cylinder.  If a read error occurs,  the routine should

simply return with an error code.  The operating system will  call

this routine up to five times to retry in case  of  errors.  After

the third retry,  the home routine will be called  to  recalibrate

the head.

4.3 Write a sector

The write routine writes a sector (512 bytes)  indicated by the

CYL,  HEAD and SEC entries in the DCB to  the  indicated  physical

drive,  from memory at the address passed  in  the  'X'  register.

Seeking and retry operation is the same as  for  the  read  sector

routine described above.

4.4 Format a disk

The format routine formats the disk in the  indicated  physical

drive with the  number  of  cylinders,  heads,  and  sectors/track

indicated by the NCYL,  NHEAD,  and NSEC values in  the  DCB.  The

sector Interleave factor is passed in the 6809 'A' register.

CUBIX System Porting Guide                                       Page: 5

5. STTY UTILITY

The complete source for  the  standard  CUBIX  'STTY'  utility  is

provided in the EXAMPLES  directory.  This  routine  is  designed  to

function with 6551 type uart devices,  and may be modified to  handle

the serial devices in the target system.

6. INTIO UTILITY

The complete source for the  standard  CUBIX  'INTIO'  utility  is

provided in the EXAMPLES  directory.  This  routine  is  designed  to

function with 6551 type uarts,  and assumes a common  interrupt  line

from all uart IRQ outputs to the 6809  IRQ  input.  If  the  standard

serial device drivers do not  support  interrupt  I/O,  this  utility

should be modified to operate in  the  hardware  environment  of  the

target system.

7. TERMINAL HANDLING

The  TTYPATCH  utility  is  provided  to   modify   the   terminal

charactistics of screen oriented programs such as  the  'EDT'  screen

editor. See the 'TTY' manual, and the 'TTYPATCH' documentation in the

CUBIX users guide for details.

CUBIX System Porting Guide                                       Page: 6

8. MAJOR PORTS

When performng a MAJOR port of  the  CUBIX  system,  you  will  be

provided with "porting" source files for the operating system and its

utilities.  The "porting" sources have all comments stripped out, and

all symbols which do not need to be modified as part of the port  are

changed to meanless alphabetic sequences.

The "porting" sources have been made purposly difficult to modify,

in order to minimize the alterations to  CUBIX,  and  subsequent  non

standard releases.  Please resist the  temptation,  as  compatability

must be maintained between CUBIX releases  in  order  for  it  to  be

successful.

8.1 Operating System

The following symbols must be set in the CUBIX  "MAIN"  source,

to adapt it to the system memory map.

The symbol  'ROM'  must be set to the starting address  of  the

system ROM  (8K rom is required).  The ROM must control  the  6809

vectors which are located at the top of the memory map at  address

$FFF2. The usual address of 'ROM' is $E000.

The symbol  'RAM'  points to a 1K block of memory which is used

by CUBIX for internal variables, buffers and stack.  It is usually

set to the top 1K of Random Access Memory,  which is $DC00 in  the

standard system.

The symbol  'USRRAM'  points to the lowest memory address which

is available for general use.  It set to  $2000  in  the  standard

system.

The symbol 'USREND'  indicates the highest memory address which

is to be tested by the startup memory test.  It is usually set  to

RAM+1023,  but is available separatly in case  OS  memory  is  not

contiguous with application memory.

The driver routines should  be  included  in  the  source  file

immediatly preceding the 6809 hardware vectors  (ORG  $FFF2)  near

the end of the file.

CUBIX System Porting Guide                                       Page: 7

8.2 Applications and Utilities

All application and utility programs must be assembled  to  fit

the memory map of the system.  A  'macros'  file is provided which

must be included at  the  beginning  of  each  source  file.  This

provides the 'SSR' macro which implements the CUBIX System Service

Request instruction.  This macro must be  modified  to  suit  your

particular macro assembler. Also provided in the 'macros' file are

the following symbols used by the application and utility sources.

The symbol  'OSRAM'  must be set to the lowest  memory  address

which is available for general use.  Is it set  to  $2000  in  the

standard system

The symbol  'OSEND'  must be set to the highest memory  address

which is available for general use.  It is set  to  $DBFF  in  the

standard system.

The symbol  'OSUTIL'  indicates the top three Kbytes of general

use memory.  This is  the  location  reserved  for  CUBIX  utility

programs which may be executed from within an application.  It  is

set to $D000 in the standard system.

The following the a sample  'macros'  file which is  compatable

with the my XASM Development Tools macro processor  and  assembler

(ASM09).

OSRAM   EQU    $2000       APPLICATION RAM AREA

OSEND   EQU    $DBFF       END OF GENERAL RAM

OSUTIL  EQU    $D000       UTILITY ADDRESS SPACE

*

* MACRO TO IMPLEMENT THE SSR FUNCTION

*

* This is not needed if using CUBIX's own assembler,

* since it has an internal 'SSR' directive.

*

SSR     MACRO

\0      SWI                INTERRUPT TO OS

FCB    \1          FUNCTION CODE NUMBER

ENDMAC

CUBIX System Porting Guide                                       Page: 8

9. PORTING FROM ROM (MINOR PORT)

A minor port may be performed directly from a  CUBIX  distribution

ROM.  This ROM contains only the operating system code,  without  the

I/O drivers.

When performing a system port from a  ROM,  the  distribution  rom

should first be copied, and all changes made on the copy. This leaves

the master rom unchanged and safe from accidental damage.

The first few locations in the distribution rom have the following

contents:

E000 FF FF       CHKSUM FDB     $FFFF   CHECKSUM GOES HERE

E002 10 00       MBASE  FDB     $2000   BASE RAM ADDRESS

E004 10 8E ?? ?? INIT   LDY     #IRAM   POINT TO INITIALIZED RAM

E008 BD XX XX           JMP     HWINIT  EXECUTE USER INIT. ROUTINE

E00B XX XX XX    SSR    EQU     *       SSR HANDLER ROUTINE

The 16 bit value at offset 0  (Address $E000)  must be set to  the

checksum of the ROM,  which is the sixteen bit sum of all ROM  memory

locations from $E002 (Offset 2) to $FFFF (Offset 8191). This checksum

must be calculated AFTER the drivers  are  installed  AND  any  other

modifications to the ROM contents have been are completed.

The 16 bit value at offset 2  (Address $E002)  identifies  to  the

operating system the lowest available RAM location.  This is used  to

determinate where to begin the memory test,  and for the default load

address of CREATE'd files.

The 'JMP'  instruction at offset 8  (Address $E008)  addresses the

user supplied 'HWINIT' routine.  In the distribution ROM,  this jumps

to the first free memory location in the ROM following the  operating

system code.  The address of  this  location  can  be  determined  by

reading the address from the  'JMP'  instruction which is at offset 9

and 10 (Address $E009 and $E00A) in the ROM.

The I/O drivers should be assembled and placed  into  the  rom  at

this address, with the HWINIT routine being the first routine so that

it will be properly entered when the  'JMP'  executes.  All remaining

rom space  up  to  the  6809  hardware  vectors  (Address  $FFF2)  is

available for use by the driver routines.

Since the operating system accesses all other driver routines only

via the table filled in by 'HWINIT',  only 'HWINIT' must be placed at

a known location (As determined earlier).

The 'SWI' vector filled in the the HWINIT routine should be set to

point to the SSR handler which begins at address $E00B.

If the base (lowest)  ram location of the target system is not the

CUBIX standard  ($2000),  the application and utility programs  which

load at the bottom of ram will have to be re-generated to reflect the

new load address (Partial MAJOR port).

CUBIX System Porting Guide                                       Page: 9

9.1 Porting from an existing system ROM

The CUBIX system may be ported from  any  existing  CUBIX  ROM,

provided that the location of the ROM, and the highest RAM address

are the same in the existing and  target  systems.  This  will  be

$E000 and $DFFF in most systems.

Ports for systems in which the ROM location or the highest  RAM

address differs from the above must be performed by  re-generating

the CUBIX image with the new memory map (MAJOR port).

If the base  (lowest)  RAM location differs from  the  existing

system to the target system,  the application and utility programs

which load at the bottom of ram will have to  be  re-generated  to

reflect the new load address (Partial MAJOR port).

9.2 Applications and Utilities

If you have the CUBIX version of the minor  port  package,  you

will have a CUBIX format disk containing the  system  applications

and utilities. In this case, the programs are ready to go, and you

may use them directly from the disk.

The HOST version of the  CUBIX  minor  port  package  does  not

include a CUBIX format disk,  but instead has the ".HEX"  download

files for each of the CUBIX system programs.

Once you have ported the CUBIX rom,  you may use it to format a

disk,  and  download  the  system  software.  For  information  on

FORMATTING and DOWNLOADING, see the CUBIX user guide.

CUBIX System Porting Guide                                       Page: 10

10. SAMPLE I/O DRIVERS

The EAMPLES directory contains a sample source  file  for  a  very

simple set of CUBIX device drivers. The drivers are fully functional,

and require minimum hardware support  (ie:  no  interrupts  are  used

etc).

This system has two 6551 serial uart devices,  and a NEC 765  type

floppy disk controller.  The 6551 drivers are fully  compatable  with

the standard STTY and INTIO utilities (Also provided in EXAMPLES).

The two 6551 uarts occur in the 6809 memory map at addresses $0000

and $0100.  For simplicity in this example,  the serial  I/O  drivers

operate by software polling the uart.  However,  the IRQ outputs from

the 6551's would normally be connected in a wired OR configuration to

the 6809 IRQ input, allowing the INTIO utility to used. In a more I/O

intensive system, it may be desirable to directly implement interrupt

driven serial drivers.

The floppy disk controller is  a  NEC  765  or  compatible,  which

controls up to 4 standard 40 track single or  double  sided  diskette

drives.  Like the serial I/O drivers, all operations are performed by

software polling.  The 765 is accessed  at  address  $0200,  and  its

TERMINAL COUNT pin is pulsed by reading or writing at address $0300.

CUBIX System Porting Guide

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

2. HARDWARE INITIALIZATION                                             2

2.1 Initializing CUBIX                                              2

2.2 Initializing Hardware Devices                                   3

2.3 Initializing the Console                                        3

3. SERIAL DEVICE DRIVERS                                               3

3.1 Input Drivers                                                   3

3.2 Output Drivers                                                  3

3.3 NULL Device Driver                                              3

3.4 Similar Devices                                                 3

4. DISK DEVICE DRIVERS                                                 4

4.1 Home head                                                       4

4.2 Read a sector                                                   4

4.3 Write a sector                                                  4

4.4 Format a disk                                                   4

5. STTY UTILITY                                                        5

6. INTIO UTILITY                                                       5

7. TERMINAL HANDLING                                                   5

8. MAJOR PORTS                                                         6

8.1 Operating System                                                6

8.2 Applications and Utilities                                      7

9. PORTING FROM ROM (MINOR PORT)                                       8

9.1 Porting from an existing system ROM                             9

9.2 Applications and Utilities                                      9

10. SAMPLE I/O DRIVERS                                                10