R.A.I.D.

(R)untime (A)id for (I)nteractive (D)ebugging

A 6809 Debugger

Revised: 30-Jul-90

Copyright 1983,1990 Dave Dunfield

All rights Reserved

RAID                                                             Page: 1

1. INTRODUCTION

RAID is an interactive debugging facility for  the  motorola  6809

microprocessor. It contains display/alter memory/register facilities,

as well as a full 6809 disassembler, breakpoint,  and software single

step functions. Download and transparent communication modes are also

included.

The format of the RAID command is as follows:

'RAID/[<device>] [<filename>]'

If <filename> is specified,  RAID first loads that file,  and sets

its saved program  counter  (PC)  to  its  starting  address.  If  no

<filename> is given,  raid goes directly to it's 'RAID>' prompt,  and

awaits your commands.

If the '/<device>' qualifier is given,  RAID communicates with the

specified serial device (0-7). IF not given, RAID assumes the current

console device.  Note that  once  RAID  has  been  invoked,  it  will

continue to communicate via the device that was console at  the  time

that  it  was  started,  even  if  the  DOS  console  assignment   is

subsequently changed.

RAID operates on each command character as soon as  it  is  typed,

there is no need to press return following command entry.  Note  that

all hex values must be entered as two or four digits.

RAID                                                             Page: 2

2. USING RAID

2.1 RAID Commands

The following commands are implemented in RAID:

B <breakpoint#> <address>

Set breakpoint at specified address. Breakpoint is removed

if address is 0000.  There can be up to eight breakpoints,

which are referenced by the numbers 0 to 7.

C <register> <value>

Changes  6809  registers  values.  Register  is  a  single

character, which may be as follows:

A   - Set A accumulator (8 bit value).

B   - Set B accumulator (8 bit value).

C   - Set condition code register (8 bit value).

D   - Set direct page register (8 bit value).

X   - Set X register (16 bit value).

Y   - Set Y register (16 bit value).

U   - Set user stack pointer (16 bit value).

S   - Set system stack pointer (16 bit value).

P   - Set program counter (16 bit value).

sp  - (space) Set D accumulator (16 bit value).

D <start>,<end>

Disassembles memory,  starting at  indicated  address.  If

space  is  entered  for  <end>  address,   assumes   FFFF.

Disassembler output contains address, opcodes bytes, ASCII

equivalent of  opcode  bytes,  instruction  neumonic,  and

operands  to  instruction.  Disassembler  output  can   be

aborted by pressing the <ESC>  key.  The line feed key may

be used to suspend output to the screen,  after which each

time line feed is pressed,  one additional  line  will  be

output.  Pressing the  carriage  return  key  will  resume

normal output.

E <address>

Edit's memory,  Address and contents are displayed,  Enter

two hex digits to change value, or a single quote followed

by a  character.  Entering  a  space  skips  to  the  next

location,  Backspace backups  to  the  previous  location.

Carriage return terminates the edit command.

G <address>

Begins execution at the indicated address.  If a space  is

entered instead on an address,  begins  execution  at  the

address in the saved 6809 program counter.

RAID                                                             Page: 3

M <start>,<end>

Displays memory, in HEX/ASCII dump format, starting at the

indicated  address.  If  a  space  is  entered  for  <end>

address,  assumes FFFF.  The output of the memory  command

can be aborted by pressing the  <ESC>  key,  line feed and

carriage return may be used to suspend output.

R

Displays the values of the 6809 registers.

S <address>

Begins single-stepping at  the  indicated  address.  If  a

space is entered for  the  address,  then  single-stepping

begins at the address in the saved 6809  program  counter.

Disassembled instruction is displayed on the terminal, and

RAID waits for a key to be pressed. If a space is entered,

steps to the next instruction. Carriage return toggles the

automatic display of registers.  '?' displays the register

values, and escape terminates the single-step command.

V

Displays the current breakpoint settings.

RAID                                                             Page: 4

2.2 Breakpoints

RAID allows breakpoints to be placed in the program under test,

which when encountered during the execution of that program, cause

the program to interrupt,  and RAID  is  entered.  Up  to  8  such

breakpoints (numbered from 0 to 7) may be placed in the program at

any one time.

When entered from a  breakpoint,  RAID  will  first  display  a

message  indicating  the  number  of  the  breakpoint  which   was

encountered, and then enter command mode.

Once RAID has been entered from a breakpoint,  the registers of

the program under test are saved.  Note that the  address  of  the

breakpoint is in the saved 6809  program  counter,  and  therefore

using a space as the operand to a 'G'  or  'S'  command will allow

execution or single stepping to continue from the breakpoint.

Breakpoints are entirely transparent,  and will remain  in  the

program until explicitly removed.

RAID                                                             Page: 5

3. WARNINGS

When executing breakpointed instruction,  or when single stepping,

RAID copies the  instruction  into  it's  own  RAM  locations  before

executing it.  Because PC relative instruction depend on  the  actual

6809 program counter being at their address in the code,  PC relative

instructions will not do proper addressing if they are  breakpointed,

or single stepped.

When RAID is invoked it attaches itself to the operating system in

such a way that it becomes an essential part of the operating system.

If for any reason RAID is terminated abnormally,  it is advisable  to

reboot the operating system.

RAID

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

2. USING RAID                                                          2

2.1 RAID Commands                                                   2

2.2 Breakpoints                                                     4

3. WARNINGS                                                            5