Summary of MX-11 functions.

---------------------------

Initialization functions

------------------------

defpoll(taskh,pollq,msg)           add task to poll list.

deftask(&task, stack_size, name)   initialize task.

mxinit(stack_size,num_tcb,num_mcb) initialize MX-11.

Timer functions

---------------

gettime()                     get hardware timer.

getTOD(time)                  get time of day.

setTOD(time)                  get time of day.

sleep(timeout)                pause task for timeout.

Message Functions

-----------------

chkmsg()                      check if message available.

flushmsg()                    flush all messages for task.

getmsg()                      get next message from queue.

sendmsg(taskh,msg)            send message to task.

sendmsgd(timeout,msg)         send message to self after delay.

Semaphore functions

-------------------

sclose(&sema)                 close semaphore.

sopen(initial_value)          get handle to semaphore.

ssignal(&sema)                signal semaphore.

swaittim(&sema,timeout)       wait on semaphore.

Serial port functions

---------------------

Cgetc()                       get character from serial port.

Copen(inbufsize,outbufsize)   initialize serial I/O.

Cprintf(format_string,args..) printf to serial port.

Cputc(c)                      print character to serial port.

Cputstr(string)               print string to serial port.

Ctestc()                      test for character at serial port.