MX-11: A Multitasking Executive for the MC68HC11.

Version 1.0

Kevin J White

12/12/93

Introduction

MX-11 provides a pre-emptive multitasking environment for the

Motorola MC68HC11 that can be used with programs written using

Dunfield Development Systems' MICRO-C/11.

I created MX-11 to simplify the task of writing software for data

acquisition and house automation projects.  I wanted to be able

to write modular software in a high-level language that could

retrieve data from sensors or switches, operate on the data and

then feed the data to displays or control other peripherals.

MX-11 does this by providing a library of C callable functions

that allow multiple operations to be performed on one processor

in an event-driven manner.  Actions can be triggered by external

events or internally by timers or another software task.

I have used the software mainly on MC68HC11A8's using the BUFFALO

monitor, it can be configured for other monitors,and other

variants of the 68HC11 by modifying the equate files.

Using MX-11

MX-11 is used by defining one or more tasks in your code and then

activating them according to your needs.

A task is created by writing a C function, and then defining it

as a MX-11 task using the function deftask().  This task can then

be activated by passing a message from another task or at

specific time intervals by adding the task to the polling list.

When a task is running it can communicate with other tasks by

messages or semaphores.  It can also use sleep() to inhibit the

task for a specified period.  If a task terminates it can be

reactivated by another task sending it a message

Tasks have a fixed priority that is set when the task is defined.

Whenever a task is made runnable or a running task is blocked,

Page - 2

MX-11 will enable the highest priority runnable task.  A

suspended task will be reactivated when the task is unblocked by,

for example a timer expiring or a semaphore being set.

MX-11 takes about 1.5-2.5k of memory space (depending upon which

functions you link in).  There are some areas where it could be

shrunk a bit.  For most of the applications I have had so far it

has not been a critical problem.

Compiling your program with MX-11.

In order to use the MX-11 functions they must be made available

to the MICRO-C source linker.  This is done by putting the

library files in the appropriate directory (usually \mc\lib11),

modifying the index file so that MICRO-C knows in which file the

functions are placed and then compiling and assembling the source

in the normal fashion to produce a hex file for download to the

target processor.

Running your Program.

The hexfile should be transferred to the target using a terminal

emulation program and a monitor such as BUFFALO or MON11.  When

the program is started the mxinit() function will set up the data

structures for MX-11 and output compare 1 for its real-time

clock.  The MX-11 function are then available for use.

Installation.

MX-11 should be installed on a machine that has the Dunfield

Development Systems Micro-C MC68HC11 compiler already in place.

I have used it with both version 2.14 and 3.0 although the examples

use structs which are not provided in version 2.14  The following

steps should be performed:

1:   Copy all of the files from the "obj" directory into your

LIB11 directory (usually c:\mc\lib11).  These files are the

sources that have been processed by sconvert.  For debugging

purposes you may want to copy the files directly from the

"src" directory so that the comments are included in the

linked output.

2:   Merge the file "MXINDEX.LIB" into the file "EXTINDEX.LIB"

which is usually in \MC\LIB11.  (This is so SLINK can find

the MX functions).  If you are using MON11 rather than the

BUFFALO monitor you should edit the MXVECT.EQU file.

Edit and compile your Micro-C programs in the normal fashion.

The MX-11 code will be linked with your software by the MICRO-C

source linker.

Page - 3

The example programs show the basic methods of using MX-11.

EX2.c has a monitor section that displays the MX-11 variables

displaying the status of the various tasks.  You could include

that code in your own programs.  Since MX-11 is multitasking the

monitor can be active simultaneously with the other tasks your

program needs to perform.  You can view memory or the activities

of your software while it is running.

Notice that the header file "MX.H" must be included in your

program, and "mxinit()" called before you use any of the MX-11

functions.

Hardware Configuration

My designs have all used a MC68HC11A8 with the BUFFALO monitor

(version 2.5 in ROM or 3.4 in non-volatile RAM).  For the

development of MX-11 I used a system with 32k of SRAM from $0-

$7FFF and 32k of non-volatile RAM from $8000-$FFFF (this is a 32k

RAM in a DALLAS Smartsocket).  I use BUFFALO version 3.4 with the

registers allocated at $1000.  Programs (Including the example

programs) start at $1040.

MX-11 uses output compare 1 for its real-time clock to give the

main system tick at 20Hz.  It assumes an 8MHz clock.  If the

clock is different you will need to adjust the divide ratio

defined by TM1CNT in MX.EQU.  (The counter actually runs at 40Hz

and divides by 2 because the maximum time for the counter is 32

milliseconds).

The SCI and its interrupt vector is also used by the serial port

driver.  No other hardware specific resources are used.  Note

that BUFFALO uses 0C5 and the XIRQ for its trace function.

The addresses of the interrupt vectors are defined in the

MXVECT.EQU file.  Edit this if your system uses has the interrupt

vectors at a different location.

MX-11 could run out of ROM.  The only function that would need

modification is the MXSTKCHK() function that patches the core

task dispatcher.  The stack check function is mainly for debug

purposes so this should not be a problem.

How does MX-11 work?

The core parts of MX-11 are in the file MXKERNAL.ASM.  The

central routine is MXSWCH: the task switcher.  Whenever a

significant event, such as the expiration of a timer, or any of

the MX-11 function calls are made, they exit through MXSWCH.  It

then searches through all the tasks that have been defined and

starts up the highest priority one that is runnable.

Page - 4

Information about athe task is contained in the Task Control

Block (TCB) that is allocated for every task when it is defined.

The address of the TCB is returned by deftask() when a task is

defined.  It should be saved in a variable as many of the MX-11

functions require that you pass the TCB of the target task.

When mxswch is called the current task has it's context stored on

the stack as if it had been interrupted.  MX-11 can then use an

RTI to resume a task.  It also saves the contents of the ?PARM

and ?TEMP locations that are used by the MICRO-C library.

The current status of a task is maintained in the "tstat" record

of the TCB.  A task can be dormant, sleeping, waiting on a

semaphore etc.  (The various states are defined in MX.EQU).  Some

of the function calls modify the state of a task.  For example

the sleep() call puts the task into a sleep state and also sets

up a timer to reawaken the task after a specified interval.

MX-11 sets up OC1 as a real time clock to interrupt every 25

milliseconds, on alternate ticks the timer functions are

processed.  The timers use another important data structure, the

Message Control Block (MCB).  A pool of these structures is

created in the heap at initialization time and they are used for

messages, timers, semaphores and the pollqueue.

When a timer is created the differential time relative to other

timers is calculated and the MCB is inserted into the list of

timers.  Normally only the head of the timer chain has to be

processed.  When it does expire the other MCB's are inspected in

case any timers are coterminous.  At expiration the command that

was stored with the MCB is executed: eg. send a message or awaken

a task.

Another use of the MCB is for defining the pollqueue.  A task can

be linked onto one of four pollqueues.  Every task is sent a

defined message at the interval for that queue.  Currently there

are queues defined for 1/20 second, second and minute polling.

Messages and semaphores also use MCB's, they are linked in a

chain to the destination task.  When the message is received or

the semaphore is closed the MCB is returned to the pool.

Hints for writing software using MX-11.

Writing code for use in a multitasking environment requires that

you be aware of the activity of all tasks. Any particular task

may be paused and another start executing at any moment.  Be

especially careful of global variables or transferring data from

one task to another in pieces larger than the processor can

handle in an atomic operation.  If multiple tasks need access to

Page - 5

a common resource then you should use a semaphore to control

access.  Cputstr() and Cprintf() use one to ensure that one task

does not interrupt data coming from another task.  You can also

disable interrupts, but this prevents any other tasks from

executing and is not recommended.

Be careful if you are writing ISR's:  They execute using the

stack of the task they interrupt and if they become blocked by

MX-11 they could cause a deadlock.

If an interrupt service routine changes the state of any task it

needs to allow "mxswch" to run.  This can be by exiting the ISR

through "mxswchi" or by calling another MX-11 function that will

do it.  If there was no change to any task status then it is

faster to do a normal RTI exit.  As an example look at the code

for the serial port in MXCOMM.ASM to see how I use semaphores

within an ISR (These ISRs don't call mxswchi since the call to

ssignal already does that).

Future Development.

The current version of MX-11 has enough functionality for use in

many projects.  There are areas where its capabilities could be

enhanced, I intend to add additional features as needs become

evident.

Currently I am working on improving the handling of task

priorities, adding queues for inter-process communication and a

library module to allow easy time manipulation for use in a

script driven home-controller project.

Please contact me if you find bugs or if there are any features

that you would find useful.  I would also like to know if you

successfully use the software.  Good luck.

Kevin J. White.

337 Pennsylvania Avenue

Los Gatos

CA 95030

Internet address kevin@novell.com

Page - 6

Summary of MX-11 functions.

---------------------------

Initialization functions

------------------------

defpoll(taskh,pollq,msg)           add task to poll list.

deftask(&task, stack_size, name)   initialize task.

mxinit(stack_size,num_tcb,num_mcb) initialize MX-11.

Timer functions

---------------

gettime()                     get hardware timer.

getTOD(time)                  get time of day.

setTOD(time)                  set time of day.

sleep(timeout)                pause task.

Message Functions

-----------------

chkmsg()                      check if message available.

flushmsg()                    flush all messages for task.

getmsg()                      get next message from queue.

sendmsg(taskh,msg)            send message to task.

sendmsgd(timeout,msg)         send message to self after delay.

Semaphore functions

-------------------

sclose(&sema)                 close semaphore.

sopen(initial_value)          get handle to semaphore.

ssignal(&sema)                signal semaphore.

swait(&sema,timeout)          wait on semaphore.

swaittim(&sema,timeout)       wait on semaphore with timeout.

Serial port functions

---------------------

Cgetc()                       get character from serial port.

Copen(inbufsize,outbufsize)   initialize serial I/O.

Cprintf(format_string,args..) printf to serial port.

Cputc(c)                      print character to serial port.

Cputstr(string)               print string to serial port.

Ctestc()                      test for character at serial port.

Page - 7

CGETC

PROTOTYPE:

char Cgetc ()

ARGUMENTS:

none.

RETURN VALUE:

character from serial port.

DESCRIPTION:

The "Cgetc" function obtains a character from the serial port.

Copen must have been called to initialize the serial port.

If there is already a character in the input buffer, the function

will return immediately.  If not the task will block waiting for

one.  Multiple tasks can wait simultaneously, although it will be

ambiguous which task receives which character.

EXAMPLE:

c=Cgetc(); /* Get a character from the serial port */

Page - 8

CHKMSG

PROTOTYPE:

int chkmsg()

ARGUMENTS:

None

RETURN VALUE:

Zero if none available.

One if at least one message available.

DESCRIPTION:

This function checks for availability of a message without

blocking the task.

EXAMPLE:

if (chkmsg()) printf("Message is %d\n", getmsg());

else printf("No Message Available\n");

Page - 9

COPEN

PROTOTYPE:

Copen(int in_buffer_size, int out_buffer_size)

ARGUMENTS:

in_buffer_size      Size of input buffer to allocate (bytes).

out_buffer_size     Size of output buffer to allocate (bytes).

RETURN VALUE:

0 for success

-1 for failure.

DESCRIPTION:

The "Copen" initializes the interrupt driven serial I/O driver.

It allocates buffer space from the heap for circular input and

output buffers.  It is assumed that the monitor program will have

initialized the baud rate, and this is not changed (BUFFALO sets

it to 9600bps).

This function must be called before any of the communication

functions are called.  The driver is interrupt driven and uses

semaphores to block tasks waiting for data transmission or

reception.  The communication functions are provided in module

MXCOMM.ASM.

EXAMPLE:

Error=Copen(8,80);/* byte input buffer, 80 byte output buffer */

Page - 10

CPRINTF

PROTOTYPE:

Cprintf(char *format_string, arg, arg,..)

ARGUMENTS:

Same as standard printf()

RETURN VALUE:

None

DESCRIPTION:

The "Cprintf" function prints to the serial port as in the

conventional printf.  A semaphore is used to ensure that the

entire string will be printed as one stream, not being

interspersed with characters from other tasks.

If the serial transmit buffer is full or another task has the use

of the port, then the task will be blocked.

Cprintf() passes the number of parameters in the D register, it

must be declared as a register function.  This can be done by

including mx.h in your program.

EXAMPLE:

Cprintf("This is some text to printf and a number %d\n",j);

Page - 11

CPUTC

PROTOTYPE:

Cputc (char c)

ARGUMENTS:

c              character to print.

RETURN VALUE:

none

DESCRIPTION:

The "Cputc" function prints one character to the serial port.

Copen must have been called to initialize the serial port.

If the transmit buffer has space the character is put into the

buffer and the function returns.  If the buffer is full, the task

is set waiting on a semaphore and will be blocked.  If multiple

tasks attempt to transmit simultaneously, the characters will be

transmitted in the order of function calls, regardless of the

priority of the calling task.  If the buffer is full there may be

multiple tasks waiting on the semaphore.

EXAMPLE:

Cputc('A'); /* Output the letter 'A' to the serial port */

Page - 12

CPUTSTR

PROTOTYPE:

Cputstr(char *s)

ARGUMENTS:

char           pointer to a string

RETURN VALUE:

none

DESCRIPTION:

The "Cputstr" function prints a string to the serial port.  A

semaphore is used to ensure that the entire string will be

printed as one stream, not being interspersed with characters

from other tasks.

If the serial transmit buffer is full or another task has the use

of the port, then the task will be blocked.

EXAMPLE:

Cputstr("This is some text to print\n");

Page - 13

CTESTC

PROTOTYPE:

int  Ctestc()

ARGUMENTS:

None

RETURN VALUE:

0-255          Character from serial port.

-1             No character available.

DESCRIPTION:

Ctestc() tests for a character at the serial port.  If none

available then -1 is returned.  If a character is available its

value is returned.  This function does not block if no character

is available.

EXAMPLE:

if ((i = ctestc())==-1) printf("No character available\n");

Page - 14

DEFPOLL

PROTOTYPE:

defpoll( char *taskh, int pollq, int msg)

ARGUMENTS:

taskh          task handle.

pollq          queue to attach task to.

msg            integer message to send to task at each wake up.

RETURN VALUE:

Zero if error, one if no error.

DESCRIPTION:

The "defpoll" function attaches a task to a poll queue to be sent

a message at regular intervals.  If the task is dormant it is

woken with the message available as its argument.  If it is awake

the message is queued for the task to retrieve it by a getmsg()

call or the the task will be re-awoken when the task becomes

dormant.  An error will be returned if an MCB cannot be obtained.

The implemented polling queues currently are:

0    Once per tick (1/20 second)

1    Once per second

2    Once per minute

EXAMPLE:

char task1h;/* Handle for task1 */

task1()

{

...

}

task1h = deftask(&task1, 128, "Task One");

defpoll(task1h, 1, 255); /* Send value of 255 once per second */

Page - 15

DEFTASK

PROTOTYPE:

char *deftask( char *function, int stack_size, char *name)

ARGUMENTS:

function       address of function to be defined as a task

stack_size     number of bytes on stack to reserve for task

name           pointer to name of task

RETURN VALUE:

Pointer to Task Control Block for task().

DESCRIPTION:

The "deftask" function allocates a Task Control Block for the

task and initializes it with starting address and stack.  The

task is set into a dormant state.  Tasks are assigned priority in

ascending order of their definition so define low priority tasks

first.  A task is started by sending a message to it or putting

it on a poll queue.

The address of the TCB for the task is returned by deftask.  The

value should be stored in a variable for use as a task handle in

later MX-11 calls eg. to send a message to it.

EXAMPLE:

char *task1h; /* Handle for task1 */

task1()

{

}

task1h = deftask(&task1, 128, "Task One");

/* Provide for 128 bytes of stack and give it a name */

sendmsg(task1h,0); /* Start it */

Page - 16

FLUSHMSG

PROTOTYPE:

flushmsg()

ARGUMENTS:

none

RETURN VALUE:

none

DESCRIPTION:

This function discards all messages pending on the current task's

message queue.

EXAMPLE:

flushmsg();  /* Discard messages */

Page - 17

GETMSG

PROTOTYPE:

int getmsg()

ARGUMENTS:

None

RETURN VALUE:

Value of message (Task blocked until message available).

DESCRIPTION:

Retrieve next message of current task's message queue, if none

available the task is blocked until one is available.

The message can be sent by sendmsg() or defpoll().

EXAMPLE:

int message;

message = getmsg();/* Obtain next message, wait until available*/

Page - 18

GETTIME

PROTOTYPE:

int gettime()

ARGUMENTS:

None

RETURN VALUE:

Value of 68HC11 hardware timer.

DESCRIPTION:

Get the value of the hardware timer.  With an 8MHz clock this

timer does two counts per microsecond.  This function is useful

for timing very short events.  Since the maximum time is less

than one system tick, the timer will wrap between clock ticks.

EXAMPLE:

int i,j;

i=gettime();

...            /* Do function you wish to time */

j=gettime();

printf("Function takes %d microseconds\n",(j-i)/2);

Page - 19

GETTOD

PROTOTYPE:

int getTOD(char *time)

ARGUMENTS:

Pointer to 4 byte array to receive time of day.

RETURN VALUE:

Time of day.

DESCRIPTION:

MX-11 maintains a time of day clock with a resolution of a

twentieth of a second.  It operates as a twenty four hour clock

and rolls over at 23:59:59.  This function copies the time into

the array given as an argument.

The time is incremented every twentieth of a second, and is

updated before tasks on the pollqueue are called.

time[0]=1/20 seconds

time[1]=seconds

time[2]=minutes

time[3]=hours.

EXAMPLE:

char time[4];

getTOD(time);

printf("The time is %2d:%02d:%02d\n",time[3], time[2], time[1]);

Page - 20

MXINIT

PROTOTYPE:

char *mxinit(int stack_size, int numtcb, int nummcb);

ARGUMENTS:

stack_size     number of bytes on stack to reserve for main().

numtcb         number of Task Control Blocks to create.

nummcb         number of Message Control Blocks to create.

RETURN VALUE:

Pointer to Task Control Block for main().

DESCRIPTION:

The "mxinit" function initializes MX-11.  It must be executed

from main() before any other MX-11 functions are called.  It

allocates memory from the heap for the TCBs and MCBs, sets up the

task main() and nulltask().  Timer OC1 is used to provide a 20 Hz

real time interrupt.

Be careful about the amount of stack space that is set aside,

some library functions require large amounts for local variables.

eg. printf uses a 100 byte buffer.

MXSTKCHK can assist in avoiding stack overflow problems.

EXAMPLE:

char mainh; /* Handle for main */

mainh = mxinit(256, 8, 32); /* Provide for 8 tasks and 32 MCBs */

Page - 21

MXSTKCHK

PROTOTYPE:

mxstkchk()

ARGUMENTS:

none

RETURN VALUE:

none

DESCRIPTION:

This enables stack checking whenever a taskswitch occurs.  This

adds about 250 microseconds to each task switch.  It patches the

mxswch function so will only function correctly when loaded into

RAM.  If ROM use is desired the file mxkernal.asm should be

modified.

It should be invoked after mxinit().

When a task is defined the maximum stack extent is recorded in

the TCB, also a stack signature is placed at the maximum stack-1.

At every task switch all tasks are checked to verify whether

there is sufficient space to support the space needed for an

interrupt, and whether the stack signature has been corrupted.

If it has, a message is output via the SCI and execution

transfers to the debugger routine "monitor" as defined in MX.EQU

file.

EXAMPLE:

char mainh; /* Handle for main */

mainh = mxinit(256, 8, 32);/* Provide for 8 tasks and 32 MCBs */

mxstkchk();

Page - 22

SCLOSE

PROTOTYPE:

int sclose(char * semaphore_handle)

ARGUMENTS:

semaphore_handle    Pointer to semaphore

RETURN VALUE:

none

DESCRIPTION:

Returns a semaphore to kernal after completion of use.  Recycles

the data structure for other tasks.

EXAMPLE:

char *sema1; /* Handle for semaphore 1 */

...

if !(sema1 = sopen(0)) printf("Error opening Semaphore\n");

...

swait(sema1); /* wait for it to be signalled */

...

sclose(sema1); /* Not needed any more, recycle it */

Page - 23

SENDMSG

PROTOTYPE:

sendmsg (char *taskh, int msg)

ARGUMENTS:

taskh          task handle

msg            integer message to send to task

RETURN VALUE:

0 if error.

1 if no error.

DESCRIPTION:

This function puts a message on a task's message queue.  If the

task is dormant, the task is scheduled to be run with the message

as its input parameter.  Alternatively the task can obtain the

message by performing the getmsg() call.

Although the message is only a 16 bit variable, it can be used as

a pointer so data of any size can be passed

EXAMPLE:

char *task1h; /* Handle for task1 */

task1(msg)

{

switch (msg)

{

...

}

}

task1h = deftask(&task1, 128, "Task One");

sendmsg (task1h, 127); /* Send value of 127 to task 1 */

Page - 24

SENDMSGD

PROTOTYPE:

sendmsgd (int time_delay, int msg)

ARGUMENTS:

time_delay     delay in ticks before message sent

msg            integer message to send to task

RETURN VALUE:

Zero if error, one if no error.

DESCRIPTION:

Sends a message to self after a time delay.  This can be used to

re-awaken a task or to provide a time-out function while waiting

for a message from another task.  The value of the message can be

used to indicate the time-out cause or can be a pointer to a data

object.

A task can have multiple timers operating simultaneously.

EXAMPLE:

#define timout_message 255

sendmsgd( 100, timeout_message);

/* wake us up in 5 seconds if nobody else calls */

...

if (getmsg()==timeout_message) printf("Timed out\n");

Page - 25

SETTOD

PROTOTYPE:

int setTOD(char *time)

ARGUMENTS:

Pointer to 4 byte array containing time of day.

RETURN VALUE:

None.

DESCRIPTION:

MX-11 maintains a time of day clock with a resolution of a

twentieth of a second.  It operates as a twenty four hour clock

and rolls over at 23:59:59.  This function copies the time from

the array given as an argument.

EXAMPLE:

char time[4];

/* Set time to 13:20 */

time[0]=0;

time[1]=0;

time[2]=20;

time[3]=13;

setTOD(time);

Page - 26

SLEEP

PROTOTYPE:

sleep( int timeout)

ARGUMENTS:

timeout        Value for timeout in clock ticks (1/20 second)

RETURN VALUE:

none

DESCRIPTION:

This function causes a pause in the execution of the calling

task.  Other tasks can still execute.

EXAMPLE:

sleep (20);    /* Cause a one second pause */

Page - 27

SOPEN

PROTOTYPE:

char *sopen(int initial_value)

ARGUMENTS:

initial_value  Starting value of the semaphore

RETURN VALUE:

handle to semaphore data structure

DESCRIPTION:

This function obtains a handle to a semaphore data structure for

use with the swait() and ssignal() functions.  The initial value

can be in the range zero to 32767 and determines how many

resources can be consumed before the task will be blocked.  If it

is set to zero the task will be blocked initially.

EXAMPLE:

char *sema1; /* Handle for semaphore 1 */

if !(sema1 = sopen(0)) printf("Error opening Semaphore\n");

Page - 28

SSIGNAL

PROTOTYPE:

void ssignal(char *semaphore_handle)

ARGUMENTS:

semaphore_handle    Pointer to semaphore

RETURN VALUE:

none

DESCRIPTION:

Increments the value of a  semaphore if no tasks are waiting on

the semaphore.  If any tasks are waiting in the semaphore, the

first one on the list is unblocked and the semaphore is left at

zero.

A semaphore can be used to provide mutual exclusion of use of a

resource.

EXAMPLE:

task1()

{

swait(sema1);   /* wait for serial I/O to be available */

printf("We don't want this to be interrupted by another task\n");

ssignal(sema1); /* Somebody else can use the serial I/O now */

}

Page - 29

SWAIT

PROTOTYPE:

int swait(char *semaphore_handle)

ARGUMENTS:

semaphore_handle    Pointer to semaphore

RETURN VALUE:

none

DESCRIPTION:

MX-11 supports counting semaphores for inter-task

synchronization.  The "swait" function tests a semaphore

previously opened with "sopen", if it has a value greater than

zero, it is decremented and the task continues.  If the value is

zero, then the task is placed on the end of the list of tasks

waiting on the semaphore and the task is blocked.  Multiple tasks

can be waiting on a semaphore and will be unblocked in list order

as "ssignal" called.

A semaphore can be used to provide mutual exclusion of use of a

resource.

EXAMPLE:

task1()

{

swait(sema1);   /* wait for serial I/O to be available */

printf("We don't want this to be interrupted by another task\n");

ssignal(sema1); /* Somebody else can use the serial I/O now */

}

Page - 30

SWAITTIM

PROTOTYPE:

int swait(char *semaphore_handle, int timeout)

ARGUMENTS:

semaphore_handle    Pointer to semaphore.

timeout             timeout value in ticks.

RETURN VALUE:

one if signalled.

Zero if time out before signalled.

DESCRIPTION:

The "swaittim" operates identically to the swait function except

that a timeout value can be specified that will cause the

function to return without if the timeout expires before the

semaphore is signalled.  The return value indicates whether a

timeout or signal occurred.

The timeout value should be specified in 1/20 second intervals.

EXAMPLE:

task1()

{

#define time_out 20

if !(swaittim(sema1, time_out))

Cprintf("Semaphore timed out\n");

}