Filename PBUFFTST.DOC  Updated 30-Apr-95 by Don McKenzie.

Note ***** Program re-written on above date. Have overcome printer port

problem. Is now auto select lpt1:

I think I have overcome the faster PC problem......

PBUFFTST.EXE was written in May 1990. Users are now finding two problems

with this test program.

(1) It works only with LPT1: set at 378Hex. (Perhaps it should read the BIOS

Data Area at 0040:0008.)

(2) A 33Mhz 386 (or faster) computer is too fast for the Basic code I have

written. A buffer full, reset pressed or buffer not present error is

reported. If this same computer is dropped back to 12Mhz, the test works

fine.

I can now do 1 of 2 things.

(1) Ignore this non-critical problem until time permits me to get back to

this code. (Perhaps even a user with a bit of time can rehash the code and

send me the results so that it can be passed onto others.)

(2) Get on with this job now!!!!

I have selected the "IGNORE" option at this stage. As long as you know.

_______________________________________________________________

This Program Tests device memory sizes and load rate of my PBUFF Printer

Buffer.

Files included with PBUFFTST

PBUFFTST.DOC   This file.

PBUFFTST.EXE   Compiled EXE file.

PBUFFTST.BAS   Basic source.

Section (1)    Screen dump of main menu.

Section (2)    Screen dump of completed test on 256K buffer.

Section (3)    Additional information and other uses.

_______________________________________________________________________

Section (1)

** PBUFFTST Ver 1.1 is a public domain program by Don McKENZIE **

29 Ellesmere Cres., Tullamarine 3043 Australia. Phone 03 338 6286

Accurately tests the memory size of my PBUFF printer buffer.

This is a Z80 based Centronics type buffer supplied as a short form kit.

Buffer must be reset and printer off line or not connected.

Updated 30-Apr-95  (Configured for LPT1:)

Program can be started by [PBUFFTST n] where n equals the number of 64K pages.

Can be used as general purpose LPT1: O/P byte counter. (add 1 for 8255 latch.)

Current memory sizes are:-

64K (1), 128K (2), 256K (4), 320K (5), 512K (8)

1024K (16), 1088K (17), 1280K (20), 2048K (32)

and 4096K (64) 4 Mb.

Control Break to Exit, reset PBUFF to abort.

Input memory size in number of 64K pages [ICS=0] ?_

_______________________________________________________________________

Section (2)

4 Page(s) selected.   256 K

Written     Percentage

255 K       PBUFF full or reset pressed.

Bytes written    Bytes installed   Bytes rejected

262,140          262,144                 4

Rejected bytes should equal number of pages installed in PBUFF

These calculations have taken into account one byte that is

latched into the 8255 but has not yet been written to Drams.

Time elapsed =  83.32 seconds.   Load rate = 3146 bytes per second.

This is approximately  189 K bytes per minute.

_______________________________________________________________________

Section (3) Additional information....

I wrote PBUFFTST in compiled turbo basic to accurately test the memory sizes

of my PBUFF printer buffers. If the memory installation is correct, this

program will verify the result to the last byte.

It can be used for other purposes as well as it simply outputs "NULL" or

zero bytes to LPT1: and counts them. It looks for busy handshaking.

It will output to a printer without printing any characters as the zeros do

not print in the normal ASCII or power up mode of the printer. Not very

useful but it does give you a load timing test that will be terminated and

reported when you take your printer off line.

It can be used to test any device that connects to the LPT1: printer card.

It counts bytes until the busy line goes permanently high after an inbuilt

timing delay so that false triggering doesn't occur. It also reports a bytes

per second load rate. I have used the IBM compatible "TIMER" basic command

for timing tests. This allows the program to work with any basic or compiled

basic that runs on any MSDOS machine. The only drawback to using "TIMER" is

that it counts seconds after midnight. Not a problem until you start the

test just before midnight and complete it after midnight, so don't run it at

midnight. Go to bed.

And don't run the true basic version for two reasons.

(1) Time-out on the busy is based on a compiled version using a loop, not

the TIMER command. True basic sometimes hangs up.

(2) It's bloody slow, very bloody slow.

Can be used to test the memory size and load rate of any buffer type device.

As my PBUFF latches one byte in its 8255 chip, my calculations have been

based on this additional byte. This byte may need to be added if the device

under test doesn't latch one byte before raising its busy line.

The Section (2) screen report was done using a 3.58mhz timing crystal in

PBUFF. Faster Z80s and faster crystals will show higher load rates. Your

computer may not be able to take advantage of this. It is usually bogged

down with maths when doing CAD/CAM operations. I am currently working on

Version 4.3 PBUFF which will increase the load time by at least 50%. Using

an 8mhz PBUFF with an AT PC as the host, the average load rate is increased

to over 10K bytes per second or 600K bytes per minute. An 8mhz PBUFF may

require some fine tuning to run reliably. I have found that the Z80H (8mhz)

doesn't like running in my board. You may have to select a Z80B (6mhz) that

will run at 8mhz. A 200ns EPROM and 150ns DRAMs will run at 8mhz. A Z80B

running at 6mhz appears to be the easiest solution for a quick reliable

PBUFF.

TEST PRINTER CARD....

It can be used for measuring printer port throughput and CPU speed by wiring

up a dummy printer as follows:-

Use a DB25 male plug as a dummy connector. Connect a 1K resistor between

ground (found on pins 18 to 25) and pin 12 (PE). Connect pin 12 to pin 13

(BUSY). You will be amazed to find that speeds of from 10K to over 100K

bytes per second will be reported in this mode. Of course, writing to any

device that has no handshaking holdups really only tests the full steam

ahead output rate.

ICS....

Incremental Character Select. This provides a means of sending the ASCII

character set to LPT1: which can be printed if required. If zero is entered

at the memory size prompt, you will be asked for memory size again.

The percentage count of PBUFFTST will also check the operation of the front

panel input output board (FPIO) of PBUFF for the correct count rate of

percentage full using the two seven segment LED displays. Arithmetic

rounding will show slight errors during the incremental stage as I have used

one byte (256 variables) to cover from 0% to 100%. It's sort of like, one a

penny one cent, two a penny two cents, three a penny two cents too. Who

remembers that? Ask your grandfather. He will know what I mean.

Send me a standard rate postage stamp for a free catalogue of my printer

buffers (over 3000 now sold), and super fast low cost Z80 (PC/XT/AT/386 or

Z80 host) development kits.

** PBUFFTST Ver 1.1 is a public domain program by Don McKENZIE **

29 Ellesmere Cres., Tullamarine 3043 Australia. Phone 03 338 6286

(Business as usual, at the same address for over 20 years)

NOTE **** 29-May-92. This program was used to test my 4Mb buffer,

however I have not yet updated the text to reflect the changes. Version

5.0 PBUFF is current.